### TP80 fast Picker robot

The TP80 fast Picker is an extremely fast robot dedicated to handling operations of small parts (under 1 kg). It is particularly well-suited for packaging applications in various fields such as food, pharmaceutical, photovoltaics or cosmetics.

### Main product benefits

### High speed

Extremely fast robot with sustained 200 picks per minute (ppm) capability.

### Simple mounting

The TP80 fast Picker can be mounted on a much lighter base than parallel robots, bringing substantial cost savings and making it easier to integrate in a cell.

### High precision and rigidity

The sturdy industrial design of the TP80 makes very repeatable throughout its work envelope.

### Large work envelope

Its 800 mm reach with 100 mm stroke enables the TP80 to cover a large work envelope while maintaining high speed.

### No arm interference when used with vision

Unlike parallel robots with their multi-arm design, the TP80 does not interfere with the line of vision of the camera.

### Slim forearm

Its slim forearm enables the robot to access confined areas, for example slotted machine openings.

### All user cables running inside the arm

Electrical, pneumatic and I/O connections are available on the tool flange with all cables running inside the arm.

### High protection

IP65 rating (with optional bellows) and angled connector design bring better protection against dust and liquids.

### Main characteristics

| Model | TP80 |
| --- | --- |
| Number of degrees of freedom | 4 |
| Maximum load capacity * | 1 kg |
| Nominal load capacity | 0,5 kg |
| Reach | 800 mm |
| Repeatability | ±0,05 mm |
| Stroke | 100 mm |
| Protection class (with optional bellows*) | IP54 (IP65*) |
| Attachment methods | Wall |
| Stäubli CS8 series controller | CS8C |

* Under specific configuration, consult Stäubli.